| HACCP PLAN Hazard Analysis and Critical Control Point Plan Gippsland Green Produce Pty Ltd | RE Number: RE-NSW-3033 Doc Ref: GIP-HACCP-034 Version: 2.3 Date: July 2025 State: Tasmania |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: Gippsland Green Produce Pty Ltd

Address: 34 Station Road, Tasmania

Commodity: Fresh Apples (Gala, Fuji), Fresh Pears (Packham)

Scope: Washing, grading, sizing, waxing, packing and cool storage of fresh apples and pears for export. Applicable to all operations within registered packhouse RE-NSW-0847.

Intended use: Fresh table fruit for retail sale. Destination: Singapore, Malaysia, UAE. Consuming population: general population including children.

2. HACCP Team

Manager 34 (Quality Lead)

Operator 34A

Operator 34B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Orchard receival | Field bins of apples/pears | Fruit received at packhouse loading dock |
| 2 | Pre-cooling / dump tank | Received fruit | Pre-cooled to 4°C; hydro-cooled in chlorinated water |
| 3 | Grading / sizing | Pre-cooled fruit | Graded by colour, size, defect (Greefa optical grader) |
| 4 | Waxing (apples only) | Graded apples | Waxed with food-grade carnauba wax |
| 5 | Packing | Graded/waxed fruit | Packed in retail trays or bulk cartons |
| 6 | Metal detection | Packed cartons | Cartons passed through inline metal detector |
| 7 | Cool storage | Packed cartons | Stored at 0–2°C / 95% RH; ethylene-controlled |
| 8 | Dispatch | Cartons from cool store | Loaded to refrigerated transport under chain of cold |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Chem | Pesticide residues exceeding importing country MRL | 3 | 3 | 9 | CCP-1 | See HACCP Control Chart |
| 1 | Bio | Codling moth (Cydia pomonella) live larvae | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 2 | Bio | Bacterial pathogens (Listeria) from dump water | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 3 | Phys | Glass / hard plastic from packhouse lighting / equipment | 1 | 3 | 3 | — | GMP / PRP control |
| 5–6 | Phys | Metal contamination from packing equipment | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |
| 7 | Bio | Botrytis grey mould growth in cool store | 2 | 2 | 4 | — | GMP / PRP control |
| 7–8 | Phys | Temperature abuse — cold chain break | 2 | 3 | 6 | CCP-5 | See HACCP Control Chart |
| 1 | Chem | Heavy metal contamination (orchard soil) | 1 | 3 | 3 | — | GMP / PRP control |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Orchard Receival | Pesticide residue MRL | Spray diary review; NATA residue testing of consignment samples | MRL per importing country (Singapore SFA / UAE) | Spray records checked per grower per season; lab residue test on 10% of consignments | Each grower per season | Reject lot; quarantine; notify grower; notify DAFF | Food Safety Manager |
| CCP-2 | Orchard Receival | Codling moth live larvae | Visual inspection of fruit at intake; cold treatment protocol | Zero live larvae in consignment | Visual inspection 100 cartons per lot; cold treatment temperature log | Per lot | Reject consignment; notify DAFF; initiate cold disinfestation protocol | Food Safety Manager |
| CCP-3 | Pre-cooling / Dump Tank | Listeria spp. in dump water | Chlorinated dump water (50–200 ppm free chlorine); pH 6.5–7.5 | Free chlorine 50 ppm minimum; pH 6.5–7.5 | Chlorine + pH test strip every 2 hours; water changed every 4 hours | Every 2 hours | Correct chlorine level; discard non-compliant water batch; re-test | Packhouse Supervisor |
| CCP-4 | Metal Detection | Metal contamination | Inline metal detector: Fe ≥2.5 mm, non-Fe ≥3.0 mm, SS ≥3.5 mm | No metal detected above sensitivity limits | Detector sensitivity test hourly with test pieces; auto-reject of flagged cartons | Each carton (inline) | Quarantine flagged cartons; inspect; trace source of metal; re-run compliant cartons | Packhouse Supervisor |
| CCP-5 | Cool Storage / Dispatch | Temperature abuse | Continuous datalogger in each cool store cell; refrigerated dispatch vehicles | 0–2°C in store; ≤5°C in transport | Datalogger reading reviewed daily; dispatch vehicle pre-cool check | Continuous (logged) | If temp exceeds 5°C for >2 hrs: inspect product; reject if deteriorated; document | Food Safety Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

Hygiene and Sanitation Program (THF-HSP-001):
• Pre-season: full clean-down of packhouse — Greefa grader, conveyors, dump tank, wax applicator — using food-safe sanitiser (sodium hypochlorite 200 ppm).
• During season: dump tank water changed every 4 hours; brushes and pads cleaned daily.
• Personnel hygiene: mandatory handwash on entry; clean protective clothing (hairnets, gloves) required in packing area.
• Footwear: boot-wash stations at all packhouse entry points.
• Cleaning schedule: daily wipe-down of packing tables; weekly deep clean of grader lanes; signed Cleaning Register (THF-CLN-01).

[DEFICIENCY — NOT DOCUMENTED]
Waste management procedures for Gippsland Green Produce Pty Ltd are NOT included in this HACCP plan. No waste streams, disposal methods, frequencies or records are documented.

7. Pest Control (Checking Point 3.2)

[DEFICIENCY — NOT DOCUMENTED]
No formal pest control program is documented for Gippsland Green Produce Pty Ltd. No bait station map, monitoring schedule, contractor engagement or corrective action procedures are included.

8. Traceability and Integrity (Checking Point 4.1)

[DEFICIENCY — NOT DOCUMENTED]
No documented traceability system linking receival, processing, packing and dispatch for Gippsland Green Produce Pty Ltd. No lot coding, one-step-back/forward procedures, or recall protocol.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

Product flow chart: see Section 3 Process Flow Diagram.

Failed Goods Procedure (THF-FGP-001):
• Lots failing pesticide residue test: placed on HOLD; grower notified; DAFF notified; lot destroyed or returned to grower for domestic use.
• Lots failing codling moth inspection: rejected from export; cold treatment applied; re-inspected before any re-presentation.
• Metal detector rejects: carton quarantined; inspected; source identified; disposed if contaminated.
• Records: all non-conformances logged in THF-NCR register.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (THF-PSP-001):
• Export-cleared fruit stored in dedicated cool store cells (CC-3 and CC-4); not co-mingled with domestic-grade fruit.
• Cartons labelled with lot code before storage; label integrity checked before dispatch.
• Packing materials (trays, cartons) stored in sealed room; not accessible to rodents or birds.
• DAFF phytosanitary certificate issued per consignment after sampling.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 2.3 | July 2025 | Manager 34 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 34 (Quality Lead)  |  Title: HACCP Lead  |  Date: July 2025

Signature:  ________________________________