| HACCP PLAN Hazard Analysis and Critical Control Point Plan Dubbo Regional Produce Pty Ltd | RE Number: RE-NSW-3076 Doc Ref: DUB-HACCP-077 Version: 1.6 Date: July 2027 State: Northern Territory |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: Dubbo Regional Produce Pty Ltd

Address: 77 Station Road, Northern Territory

Commodity: Almonds (Nonpareil, Carmel)

Scope: Hulling, shelling, drying, sorting, grading and packing of almonds for export. Registered area RE-SA-3318, Renmark facility.

Intended use: Retail and food service almonds. Destinations: Japan, USA, Germany. Consuming population: general including allergic individuals (tree nut allergen).

2. HACCP Team

Manager 77 (Quality Lead)

Operator 77A

Operator 77B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Hull/shell receival | Field-harvested wet almonds | Almonds received at hulling intake |
| 2 | Hulling | Received almonds | Hull removed; hulls collected as by-product |
| 3 | Shelling | Hulled almonds | Shell removed; kernels separated from shell |
| 4 | Drying | Wet kernels (>8% moisture) | Kernels dried to ≤6% moisture (hot air dryer) |
| 5 | Sizing / grading | Dry kernels | Sized by screen; graded by colour (optical sorter) |
| 6 | Colour sorting | Sized kernels | Foreign objects, mould-damaged kernels removed |
| 7 | Metal detection | Sorted kernels | In-line metal detector pass |
| 8 | Packing | Inspected kernels | Packed in retail bags, bulk cartons or bulk totes |
| 9 | Cold storage | Packed almonds | Stored at 2–4°C / <65% RH |
| 10 | Dispatch | Cold-stored packs | Loaded to refrigerated transport |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–4 | Chem | Aflatoxin (B1, B2, G1, G2) from mould — critical hazard | 3 | 3 | 9 | CCP-1 | See HACCP Control Chart |
| 1 | Bio | Salmonella on outer hull/shell surface | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 4 | Chem | PAH contamination from hot air dryer (fuel combustion) | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 5–6 | Phys | Shell fragments in kernel output | 2 | 2 | 4 | — | GMP / PRP control |
| 6–7 | Phys | Metal contamination from processing equipment | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |
| 8–9 | Allerg | Cross-contact with other tree nuts (cashew, walnut) if shared facility | 2 | 3 | 6 | CCP-5 | See HACCP Control Chart |
| 9 | Bio | Bacterial growth from temperature abuse | 2 | 3 | 6 | CCP-6 | See HACCP Control Chart |
| All | Chem | Pesticide MRL (chlorpyrifos, dimethoate) | 2 | 3 | 6 | CCP-7 | See HACCP Control Chart |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Receival/Drying | Aflatoxin | NATA aflatoxin test each lot; moisture control during drying | AFB1 ≤2 µg/kg (EU); total aflatoxin ≤4 µg/kg (USA) | NATA test on every receival lot; moisture check before and after drying | Per lot at receival; post-drying | See: Failed goods procedure — [DEFICIENCY: No procedure documented for aflatoxin-positive lots] | Food Safety Manager |
| CCP-2 | Receival | Salmonella | Surface sanitation of intake equipment; equipment cleaning after each variety | Salmonella — not detected in 25 g | Environmental swab program (quarterly); process water hygiene | Quarterly swabs; per season water test | Intensify cleaning; environmental investigation; product quarantine if positive | Food Safety Manager |
| CCP-3 | Drying | PAH from dryer | Commercial ISDN-grade fuel; regular burner maintenance and calibration | BaP ≤2 µg/kg in dried product | Annual burner service; fuel quality certificate retained | Annual service; per season fuel batch | Stop dryer; service burner; test product before resuming | Processing Manager |
| CCP-4 | Metal Detection | Metal contamination | Inline metal detector: Fe ≥2.0 mm, non-Fe ≥2.5 mm | No metal above sensitivity limits | Metal detector calibrated each shift with test pieces | Each carton (inline) | Quarantine line; trace metal source; re-run compliant product | Processing Manager |
| CCP-5 | Packing | Allergen cross-contact | Dedicated almond-only lines; TACT allergen cleaning between species | No detectable cashew or walnut protein (ELISA) | ELISA test after allergen cleaning; visual inspection | After each allergen cleaning event | Re-clean; re-test; hold product | Food Safety Manager |
| CCP-6 | Cold Storage | Temperature abuse | Continuous datalogger; automatic alarm at >6°C | ≤4°C | Datalogger reviewed daily | Continuous | Investigate; reject deteriorated product; service refrigeration | Processing Manager |
| CCP-7 | Receival | Pesticide MRL | Spray diary per grower; residue test on 5% of receival lots | MRL per Japan, EU, USA (strictest applicable) | Spray records each grower each season; residue test flagged lots | Per grower/season | Quarantine; reject; notify grower | Food Safety Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

[DEFICIENCY — NOT DOCUMENSWED]
No site-specific hygiene and sanitation program documented for Dubbo Regional Produce Pty Ltd. General industry practices are referenced but no cleaning schedules, chemicals, personnel hygiene procedures or records are defined.

Waste Management Procedure (RAP-WMP-001):
• Hulls: collected and sold to stock feed market; removed within 24 hours.
• Shells: sold to garden mulch supplier; stored in sealed covered skip.
• Sorting rejects (mouldy kernels): collected in sealed bins; sent to landfill as food waste.
• Dryer exhaust: ducted to external atmosphere away from product intakes.
• Wastewater: directed to trade waste system under council approval.

7. Pest Control (Checking Point 3.2)

Pest Control Program (DUB-PCP-001):
• General pest controls described.
• No formal monitoring schedule or corrective action procedure documented.
• Pest effectiveness cannot be demonstrated without records.

8. Traceability and Integrity (Checking Point 4.1)

[DEFICIENCY — NOT DOCUMENSWED]
No documented traceability system linking receival, processing, packing and dispatch for Dubbo Regional Produce Pty Ltd. No lot coding, one-step-back/forward procedures, or recall protocol.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

[DEFICIENCY] No standalone product flow chart diagram attached to this HACCP plan.

[DEFICIENCY — NOT DOCUMENSWED]
No formal failed goods procedure for Dubbo Regional Produce Pty Ltd. No documented process for segregation, re-testing, downgrade, destruction or DAFF notification.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (RAP-PSP-001):
• Packed and inspected almonds stored in dedicated cold store rooms (CC-1, CC-2) separate from raw/unprocessed product.
• Carton labels applied in processing area; label integrity verified before dispatch.
• DAFF phytosanitary certificate issued per consignment.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.6 | July 2027 | Manager 77 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 77 (Quality Lead)  |  Title: HACCP Lead  |  Date: July 2027

Signature:  ________________________________