| HACCP PLAN Hazard Analysis and Critical Control Point Plan Wheatbelt Storage & Handling Pty Ltd | RE Number: RE-SA-3010 Doc Ref: WHE-HACCP-011 Version: 3.0 Date: June 2024 State: South Australia |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: Wheatbelt Storage & Handling Pty Ltd

Address: 11 Station Road, South Australia

Commodity: Canola ( export grade ), Linseed

Scope: Receival, storage, cleaning and bulk loading of canola and linseed for export. Cold pressing of linseed oil (in-line). Registered area RE-NSW-1882, Kwinana facility.

Intended use: Canola for crushing/oil extraction. Linseed oil for food and industrial use. Destination: Japan (canola), Germany (linseed oil). Non-GM canola for EU markets.

2. HACCP Team

Manager 11 (Quality Lead)

Operator 11A

Operator 11B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Paddock planning & variety selection | Variety/agronomy plans | Seed selection — GM-status documented |
| 2 | Harvest & field transport | Standing crop | Harvested oilseed at receival pit |
| 3 | Receival / intake testing | Bulk oilseed | Oil content, moisture, impurities tested at intake |
| 4 | Drying / conditioning | Wet grain (>9% moisture) | Dried oilseed ≤8% moisture |
| 5 | Storage | Dry oilseed | Stored in designated silo — GM and non-GM segregated |
| 6 | Cleaning / scalping | Stored oilseed | Cleaned of trash, weed seeds and foreign material |
| 7 | Cold pressing (linseed only) | Clean linseed | Linseed oil extracted; meal collected |
| 8 | Bulk loading (canola) | Cleaned canola | Loaded to bulk vessel via pneumatic conveyor |
| 9 | Tanker filling (linseed oil) | Pressed linseed oil | Filled to food-grade tanker |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–2 | Chem | Glucosinolates above export spec (canola) | 2 | 2 | 4 | — | GMP / PRP control |
| 1–2 | Chem | Erucic acid above EU limit (0.5%) — old varieties | 1 | 3 | 3 | — | GMP / PRP control |
| 1 | Chem | Pesticide MRL exceedance — herbicide, fungicide | 2 | 3 | 6 | CCP-1 | See HACCP Control Chart |
| 3–5 | Chem | Mycotoxin (Ochratoxin A, aflatoxin) from wet/damaged seed | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 5 | Bio | Insect infestation (grain weevils, beetles) in storage | 2 | 2 | 4 | — | GMP / PRP control |
| 5 | Bio | Rodent infestation in silos | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 6 | Phys | Foreign material — stones, steel, glass | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |
| 7 | Chem | PAH contamination from pressing equipment (lubricants) | 1 | 3 | 3 | — | GMP / PRP control |
| 8–9 | Phys | Transport unit/tanker contamination (previous cargo) | 2 | 2 | 4 | — | GMP / PRP control |
| 5 | Chem | GM cross-contamination of non-GM lots (Identity Preservation) | 3 | 3 | 9 | CCP-5 | See HACCP Control Chart |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Receival | Pesticide MRL | Spray diary review per grower; residue test on flag lots | MRL per Japan / EU Regulation 396/2005 | Spray records checked each grower per harvest; residue test 5% of receival lots | Per grower/season | Reject lot; quarantine; grower notified; DAFF notified | QA Manager |
| CCP-2 | Storage | Mycotoxin | Moisture monitoring; aeration; temperature probe | OA ≤3 µg/kg; aflatoxin ≤4 µg/kg | Intake moisture test; monthly temperature probe; lab test on export lots | Monthly during storage; per export consignment | Segregate affected silo; test; downgrade or destroy if above threshold | QA Manager |
| CCP-3 | Storage | Rodent infestation | 16 perimeter bait stations; monthly inspection | No visible live rodents or fresh signs in storage zone | Monthly bait station inspection (Form AOW-PC-01); pitfall traps quarterly | Monthly | Engage licensed contractor immediately; quarantine affected silo | Plant Manager |
| CCP-4 | Cleaning/Scalping | Foreign material | Scalper (6 mm screen), rare-earth magnetic separator | No metal > 2 mm; no glass; no hard foreign matter | Scalper screen inspection each shift; magnet cleaning log | Each shift | Stop line; remove contaminant; inspect product; re-screen | Plant Manager |
| CCP-5 | Storage/Loading | GM cross-contamination | Dedicated IP silos for non-GM; flush/cleanout procedures; GM test on non-GM lots | <0.9% adventitious GM presence (EU threshold) | PCR test on non-GM lots at receival and pre-shipment | Per non-GM lot | Reject non-GM classification; re-test; notify customer | QA Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

[DEFICIENCY — NOT DOCUMENTED]
No site-specific hygiene and sanitation program documented for Wheatbelt Storage & Handling Pty Ltd. General industry practices are referenced but no cleaning schedules, chemicals, personnel hygiene procedures or records are defined.

[DEFICIENCY — NOT DOCUMENTED]
Waste management procedures for Wheatbelt Storage & Handling Pty Ltd are NOT included in this HACCP plan. No waste streams, disposal methods, frequencies or records are documented.

7. Pest Control (Checking Point 3.2)

Pest Control Program (AOW-PCP-001):
• Rodents: 16 perimeter bait stations (Bromadiolone); monthly inspection; licensed contractor annual review.
• Insects: pitfall traps in each silo; grain temperature probe weekly in storage season; phosphine fumigation when insects detected.
• Bird exclusion: all structural openings sealed; netting at loading conveyor ends.
• Records: Pest Control Log (AOW-PC-01) monthly.

8. Traceability and Integrity (Checking Point 4.1)

Traceability System (AOW-TRACE-001):
• Each silo assigned unique ID (silo number + commodity + IP/non-GM status + season).
• Receival dockets link grower → farm block → variety → quantity → silo.
• Dispatch: bill of lading links silo ID → vessel/tanker → destination.
• GM testing records linked to lot reference; retained 5 years (EU requirement).
• One-step-forward / one-step-back trace exercise performed semi-annually.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

Product flow chart: see Section 3 Process Flow Diagram.

Failed Goods Procedure (AOW-FGP-001):
• GM contamination above threshold: lot re-classified as non-Identity Preserved; customer notified; DAFF notified.
• Mycotoxin exceedance: silo placed on HOLD; re-test from secondary sample; if confirmed, dispose to non-food use.
• Pesticide MRL: reject export consignment; segregate; grower liability documented; option to process to domestic oil (if MRL met under FSANZ).
• All non-conformances recorded in AOW-NCR register with root cause analysis.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (AOW-PSP-001):
• Export-cleared grain stored in dedicated silos; not accessible to uncleared grain.
• Identity Preserved (non-GM) lots in dedicated, clearly labelled IP silos.
• Loading procedure: transport unit inspected before loading; last cargo record reviewed; sealed containers sealed at point of loading.
• DAFF sampling completed before loading of each export lot.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 3.0 | June 2024 | Manager 11 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 11 (Quality Lead)  |  Title: HACCP Lead  |  Date: June 2024

Signature:  ________________________________