| HACCP PLAN Hazard Analysis and Critical Control Point Plan Wangaratta Vine & Grain Co | RE Number: RE-TAS-3082 Doc Ref: QLDN-HACCP-083 Version: 3.2 Date: October 2027 State: Tasmania |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: Wangaratta Vine & Grain Co

Address: 83 Station Road, Tasmania

Commodity: Canola ( export grade ), Linseed

Scope: Receival, storage, cleaning and bulk loading of canola and linseed for export. Cold pressing of linseed oil (in-line). Registered area RE-QLD-1882, Kwinana facility.

Intended use: Canola for crushing/oil extraction. Linseed oil for food and industrial use. Destination: Japan (canola), Germany (linseed oil). Non-GM canola for EU markets.

2. HACCP Team

Manager 83 (Quality Lead)

Operator 83A

Operator 83B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Paddock planning & variety selection | Variety/agronomy plans | Seed selection — GM-status documented |
| 2 | Harvest & field transport | Standing crop | Harvested oilseed at receival pit |
| 3 | Receival / intake testing | Bulk oilseed | Oil content, moisture, impurities tested at intake |
| 4 | Drying / conditioning | Wet grain (>9% moisture) | Dried oilseed ≤8% moisture |
| 5 | Storage | Dry oilseed | Stored in designated silo — GM and non-GM segregated |
| 6 | Cleaning / scalping | Stored oilseed | Cleaned of trash, weed seeds and foreign material |
| 7 | Cold pressing (linseed only) | Clean linseed | Linseed oil extracted; meal collected |
| 8 | Bulk loading (canola) | Cleaned canola | Loaded to bulk vessel via pneumatic conveyor |
| 9 | Tanker filling (linseed oil) | Pressed linseed oil | Filled to food-grade tanker |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–2 | Chem | Glucosinolates above export spec (canola) | 2 | 2 | 4 | — | GMP / PRP control |
| 1–2 | Chem | Erucic acid above EU limit (0.5%) — old varieties | 1 | 3 | 3 | — | GMP / PRP control |
| 1 | Chem | Pesticide MRL exceedance — herbicide, fungicide | 2 | 3 | 6 | CCP-1 | See HACCP Control Chart |
| 3–5 | Chem | Mycotoxin (Ochratoxin A, aflatoxin) from wet/damaged seed | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 5 | Bio | Insect infestation (grain weevils, beetles) in storage | 2 | 2 | 4 | — | GMP / PRP control |
| 5 | Bio | Rodent infestation in silos | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 6 | Phys | Foreign material — stones, steel, glass | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |
| 7 | Chem | PAH contamination from pressing equipment (lubricants) | 1 | 3 | 3 | — | GMP / PRP control |
| 8–9 | Phys | Transport unit/tanker contamination (previous cargo) | 2 | 2 | 4 | — | GMP / PRP control |
| 5 | Chem | GM cross-contamination of non-GM lots (Identity Preservation) | 3 | 3 | 9 | CCP-5 | See HACCP Control Chart |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Receival | Pesticide MRL | Spray diary review per grower; residue test on flag lots | MRL per Japan / EU Regulation 396/2005 | Spray records checked each grower per harvest; residue test 5% of receival lots | Per grower/season | Reject lot; quarantine; grower notified; DAFF notified | QA Manager |
| CCP-2 | Storage | Mycotoxin | Moisture monitoring; aeration; temperature probe | OA ≤3 µg/kg; aflatoxin ≤4 µg/kg | Intake moisture test; monthly temperature probe; lab test on export lots | Monthly during storage; per export consignment | Segregate affected silo; test; downgrade or destroy if above threshold | QA Manager |
| CCP-3 | Storage | Rodent infestation | 16 perimeter bait stations; monthly inspection | No visible live rodents or fresh signs in storage zone | Monthly bait station inspection (Form AOW-PC-01); pitfall traps quarterly | Monthly | Engage licensed contractor immediately; quarantine affected silo | Plant Manager |
| CCP-4 | Cleaning/Scalping | Foreign material | Scalper (6 mm screen), rare-earth magnetic separator | No metal > 2 mm; no glass; no hard foreign matter | Scalper screen inspection each shift; magnet cleaning log | Each shift | Stop line; remove contaminant; inspect product; re-screen | Plant Manager |
| CCP-5 | Storage/Loading | GM cross-contamination | Dedicated IP silos for non-GM; flush/cleanout procedures; GM test on non-GM lots | <0.9% adventitious GM presence (EU threshold) | PCR test on non-GM lots at receival and pre-shipment | Per non-GM lot | Reject non-GM classification; re-test; notify customer | QA Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

[DEFICIENCY — NOT DOCUMENTED]
No site-specific hygiene and sanitation program documented for Wangaratta Vine & Grain Co. General industry practices are referenced but no cleaning schedules, chemicals, personnel hygiene procedures or records are defined.

Waste Management Procedure (AOW-WMP-001):
• Scalper screenings / trash: collected in sealed bins; transported off-site to approved composting.
• Oilseed dust: cyclone dust collector; dust collected in sealed bags; removed weekly.
• Linseed meal (pressing by-product): sold to livestock feed market; removed within 48 hours of pressing.
• Chemical waste: fumigant and pesticide containers disposed per APVMA label; manifest maintained.
• Wastewater from cleaning: directed to grease trap; collected by liquid waste contractor monthly.

7. Pest Control (Checking Point 3.2)

[DEFICIENCY — NOT DOCUMENTED]
No formal pest control program is documented for Wangaratta Vine & Grain Co. No bait station map, monitoring schedule, contractor engagement or corrective action procedures are included.

8. Traceability and Integrity (Checking Point 4.1)

[DEFICIENCY — NOT DOCUMENTED]
No documented traceability system linking receival, processing, packing and dispatch for Wangaratta Vine & Grain Co. No lot coding, one-step-back/forward procedures, or recall protocol.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

[DEFICIENCY] No standalone product flow chart diagram attached to this HACCP plan.

Failed Goods Procedure (AOW-FGP-001):
• GM contamination above threshold: lot re-classified as non-Identity Preserved; customer notified; DAFF notified.
• Mycotoxin exceedance: silo placed on HOLD; re-test from secondary sample; if confirmed, dispose to non-food use.
• Pesticide MRL: reject export consignment; segregate; grower liability documented; option to process to domestic oil (if MRL met under FSANZ).
• All non-conformances recorded in AOW-NCR register with root cause analysis.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (AOW-PSP-001):
• Export-cleared grain stored in dedicated silos; not accessible to uncleared grain.
• Identity Preserved (non-GM) lots in dedicated, clearly labelled IP silos.
• Loading procedure: transport unit inspected before loading; last cargo record reviewed; sealed containers sealed at point of loading.
• DAFF sampling completed before loading of each export lot.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 3.2 | October 2027 | Manager 83 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 83 (Quality Lead)  |  Title: HACCP Lead  |  Date: October 2027

Signature:  ________________________________