| HACCP PLAN Hazard Analysis and Critical Control Point Plan New England Wool & Grain Pty Ltd | RE Number: RE-NSW-3035 Doc Ref: NEW-HACCP-036 Version: 4.5 Date: August 2025 State: New South Wales |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: New England Wool & Grain Pty Ltd

Address: 36 Station Road, New South Wales

Commodity: White Sorghum, Hard Wheat

Scope: Receival, blending, storage, cleaning and bulk loading of food-grade sorghum and wheat at Lucinda port terminal for export. Registered area RE-QLD-1547.

Intended use: White sorghum for food processing (tortillas, beer). Wheat for milling. Destinations: Mexico, Indonesia. Retail-facing end products.

2. HACCP Team

Manager 36 (Quality Lead)

Operator 36A

Operator 36B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Receival / intake weighing | Bulk grain from farm trucks | Grain tipped into receival pit; moisture, protein, screenings tested |
| 2 | Pre-cleaning | Received grain | Pre-cleaned through scalper to remove trash |
| 3 | Drying (if required) | Wet grain (sorghum >12.5%, wheat >13%) | Dried grain to spec |
| 4 | Storage | Dry grain | Grain allocated to export silos (S1–S8) by grade |
| 5 | Blending | Multiple silo stocks | Blended consignment to customer specification |
| 6 | Final cleaning / screening | Blended grain | Cleaned through gravity cleaner and de-stoner |
| 7 | Fumigation | Cleaned grain in hold silo | Fumigated grain; treated certificate issued |
| 8 | Sampling | Fumigated grain | Composite probe sample; NATA analysis |
| 9 | Vessel loading | Cleared grain | Grain conveyed to bulk vessel through covered gallery |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Chem | Mycotoxin (Fumonisins, AFB1) in sorghum at receival | 3 | 3 | 9 | CCP-1 | See HACCP Control Chart |
| 1 | Chem | Pesticide MRL exceedance | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 1–3 | Bio | Ergot in wheat at intake | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 4 | Bio | Rodent infestation in terminal silos | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |
| 4 | Bio | Insect infestation (weevils, grain moths) | 2 | 2 | 4 | — | GMP / PRP control |
| 5 | Chem | Grade/variety substitution — blending error | 2 | 3 | 6 | CCP-5 | See HACCP Control Chart |
| 7 | Chem | Phosphine fumigant residue | 2 | 3 | 6 | CCP-6 | See HACCP Control Chart |
| 9 | Phys | Vessel cargo hold contamination (previous cargo) | 2 | 3 | 6 | CCP-7 | See HACCP Control Chart |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Receival | Mycotoxin (Fumonisins/AFB1) | Intake mycotoxin rapid test; lab confirmation | Fumonisins ≤2,000 µg/kg (EU); AFB1 ≤2 µg/kg | Rapid test on each receival lot (RidaQuick strip); NATA lab confirmation on flag | Each lot at intake | Reject lot; quarantine; notify grower; DAFF notification | Quality Coordinator |
| CCP-2 | Receival | Pesticide MRL | Spray diary review; residue testing on flagged lots | MRL per Mexico / Indonesia (Codex defaults) | Spray records checked per grower; residue test 5% of lots | Per grower/season | Quarantine; reject; notify grower | Quality Coordinator |
| CCP-3 | Receival/Storage | Ergot in wheat | Sieve test at intake; visual inspection | <0.05% ergot bodies by weight | Sieve test + visual each wheat lot at intake | Each wheat receival lot | Reject lot; return to grower; document | Terminal Manager |
| CCP-4 | Storage | Rodent infestation | Bait stations — 12 stations inside terminal building | No evidence of live rodents or fresh activity | Bait stations checked but NO formal monitoring schedule or corrective action procedure documented. ⚠ DEFICIENCY: Pest monitoring frequency not specified; no corrective action procedure if infestation detected. ACTION: Document monitoring schedule (e.g., fortnightly inspection) and corrective action steps. | Monthly (target — not documented) | NOT SPECIFIED — corrective action procedure absent | Terminal Manager |
| CCP-5 | Blending | Grade substitution | Automated blending system with silo ID cross-check | Blend recipe must match customer spec; tolerance ±0.5% | Blending computer log reviewed per vessel; print retained | Per vessel loading | Halt blending; investigate; re-blend; notify customer | Terminal Manager |
| CCP-6 | Fumigation | Phosphine residue | Licensed fumigator; ventilation protocol | ≤0.01 mg/kg residue at load-out | Fumigation certificate per event; vent period checklist | Each fumigation | Extend vent period; re-test | Quality Coordinator |
| CCP-7 | Vessel Loading | Vessel cargo hold contamination | Vessel hold inspection before loading; DAFF approval | Hold clean; free from previous cargo; no moisture | Hold inspection by DAFF-authorised surveyor | Per vessel | Reject loading; re-clean hold; re-inspect | Terminal Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

Hygiene and Sanitation Program (NQGT-HSP-001):
• Terminal cleaning: receival pit cleaned weekly; conveyor gallery swept daily; elevator boots washed monthly.
• Between commodities: blow-down of all conveyors and elevator legs before changing grain type.
• Vessel loading spout cleaned after each vessel loading.
• Personnel: safety footwear and dust masks required in operational areas; handwash on entry.
• Records: Cleaning Register (NQGT-CLN-01).

[DEFICIENCY — NOT DOCUMENTED]
Waste management procedures for New England Wool & Grain Pty Ltd are NOT included in this HACCP plan. No waste streams, disposal methods, frequencies or records are documented.

7. Pest Control (Checking Point 3.2)

Pest Control Program (NEW-PCP-001):
• General pest controls described.
• No formal monitoring schedule or corrective action procedure documented.
• Pest effectiveness cannot be demonstrated without records.

8. Traceability and Integrity (Checking Point 4.1)

Traceability System (NQGT-TRACE-001):
• Receival dockets: grower → truck rego → weight → silo allocation.
• Silo inventory log: daily balance per silo.
• Vessel loading manifest: lists source silos, quantities, blending ratios, fumigation certificate ref.
• Retained 2 years.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

Product flow chart: see Section 3 Process Flow Diagram.

[DEFICIENCY — NOT DOCUMENTED]
No formal failed goods procedure for New England Wool & Grain Pty Ltd. No documented process for segregation, re-testing, downgrade, destruction or DAFF notification.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (NQGT-PSP-001):
• Vessel hold surveyed and approved by DAFF before loading.
• Phytosanitary certificate issued per vessel by DAFF.
• Post-fumigation grain not moved until DAFF sampling complete.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 4.5 | August 2025 | Manager 36 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 36 (Quality Lead)  |  Title: HACCP Lead  |  Date: August 2025

Signature:  ________________________________