| HACCP PLAN Hazard Analysis and Critical Control Point Plan Noosa Ginger & Spice Co Pty Ltd | RE Number: RE-VIC-3088 Doc Ref: NOO-HACCP-089 Version: 1.8 Date: February 2028 State: Western Australia |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: Noosa Ginger & Spice Co Pty Ltd

Address: 89 Station Road, Western Australia

Commodity: Wheat (ASW, APW), Barley (Feed/Malt)

Scope: Receival, drying, storage, cleaning/grading, sampling and bulk loading of wheat and barley for export under the Export Control Act 2020. Applicable to all export-grade grain handled within registered area RE-NSW-3051.

Intended use: Malt-grade barley for malting/brewing industry. APW1/ASW wheat for milling. Consuming population: general adult population. Destination: Japan, Vietnam.

2. HACCP Team

Manager 89 (Quality Lead)

Operator 89A

Operator 89B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Paddock selection & planning | Paddock history, soil test results | Grain lots planned for harvest |
| 2 | Harvesting | Standing crop | Harvested grain (header bin/chaser) |
| 3 | Field transport to silo | Harvested grain in chaser/cart | Grain at receival pit |
| 4 | Receival / intake | Grain at pit | Grain tipped into receival pit; probed for moisture, grade |
| 5 | Drying (if moisture >13%) | Wet grain | Grain dried to ≤12.5% moisture |
| 6 | Bulk storage | Dry grain | Grain in sealed silo with aeration |
| 7 | Cleaning / screening | Stored grain | Cleaned grain (scalper + indent cylinder) |
| 8 | Sampling | Cleaned grain | Probe sample submitted to NATA lab |
| 9 | Fumigation (if required) | Grain in storage | Treated grain; fumigation certificate issued |
| 10 | Loading / dispatch | Sampled & cleared grain | Grain loaded to transport unit; dispatch docket signed |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Chem | Fusarium mycotoxins (DON, ZON) from pre-harvest conditions | 3 | 2 | 6 | CCP-1 | See HACCP Control Chart |
| 1 | Chem | Storage mycotoxins (Ochratoxin A) — variety/storage risk | 2 | 3 | 6 | CCP-1 | See HACCP Control Chart |
| 4 | Chem | Pesticide residues exceeding MRL (importing country) | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 4–6 | Bio | Ergot (Claviceps purpurea) contamination | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 6 | Bio | Rodent infestation / contamination in storage | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |
| 6 | Bio | Insect infestation (weevils, moths) in storage | 2 | 2 | 4 | — | GMP / PRP control |
| 7 | Phys | Foreign material — glass, metal, stones | 2 | 3 | 6 | CCP-5 | See HACCP Control Chart |
| 9 | Chem | Fumigant residue (phosphine) above MRL | 2 | 3 | 6 | CCP-6 | See HACCP Control Chart |
| 10 | Phys | Transport unit contamination (previous cargo) | 2 | 2 | 4 | — | GMP / PRP control |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Planning/Intake | Mycotoxin (DON/OA) | Variety selection; field risk assessment; intake sampling | DON ≤1,250 µg/kg (EU); OA ≤3 µg/kg | NATA lab test each receival lot | Per lot at intake | Reject lot; segregate; do not blend with compliant grain | Quality Manager |
| CCP-2 | Receival | Pesticide MRL exceedance | Spray records review; MRL screening | MRL per destination country (Japan/Vietnam CODEX) | Chemical spray diary review; residue test if flagged | Per grower per season | Reject lot; notify grower; notify DAFF | Quality Manager |
| CCP-3 | Receival/Storage | Ergot | Visual sort; sieve test | Zero tolerance (EU); <0.05% (Japan) | Visual inspection at intake; NATA ergot test on export lots | Each receival lot | Segregate; downgrade; do not export | Quality Manager |
| CCP-4 | Storage | Rodent infestation | Bait stations (18 perimeter); monthly inspection; grain temperature probe | No visual evidence of live rodents or fresh droppings in storage | Monthly bait station inspection log; quarterly grain probe trap | Monthly | Increase bait density; engage licensed pest controller; quarantine affected silo | Operations Manager |
| CCP-5 | Cleaning/Screening | Foreign material (glass/metal) | Scalper screen; rare-earth magnet; indent cylinder | Metal: none detected; Glass: none detected | Scalper screen inspection daily; magnet cleaning log | Each shift | Stop line; investigate source; document; re-screen | Operations Manager |
| CCP-6 | Fumigation | Phosphine residue | Ventilation protocol post-fumigation | ≤0.01 mg/kg phosphine residue in grain at load-out | Licensed fumigator certificate; vent period checklist | Each fumigation event | Extend vent period; re-test; do not load until cleared | Quality Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

Hygiene and Sanitation Program (EF-HSP-002):
• Pre-harvest: all silo bins cleaned, swept and bait-bag tested before intake.
• Equipment: grain elevator, conveyors and scalper cleaned between commodity changes (compressed air + brush).
• Cleaning schedule: weekly sweep of receival pit floor; monthly wash of elevator boot; bi-annual pressure wash of silo interiors between crops.
• Personnel hygiene: no-food-in-grain-area policy; handwashing facilities at site entry; PPE (dust masks, gloves) provided.
• Records: Cleaning register (Form GCE-CLN-01) signed after each event.

Waste Management Procedure (EF-WMP-001):
• Screenings: collected in sealed bins; removed by licensed carrier to approved composting facility weekly.
• Grain dust: dust extraction unit emptied into sealed skip; disposed with screenings.
• Chemical waste (empty fumigant sachets, bait material): disposed per APVMA label instructions; manifest retained.
• Liquid waste (lubricants, hydraulic oil): collected in drip trays; removed by licensed liquid waste contractor.
• Waste storage area: segregated from grain storage; labelled; covered to prevent vermin access.

7. Pest Control (Checking Point 3.2)

Pest Control Program (EF-PCP-001) — Integrated Pest Management:
• Rodent control: 18 perimeter bait stations (Bromadiolone, tamper-resistant boxes), mapped in Bait Station Plan.
• Insect control: phosphine fumigation of stored grain when insect activity detected by pitfall traps.
• Bird exclusion: all silo vents and openings bird-proofed; netting installed at conveyor gallery ends.
• Monitoring: monthly visual inspection of all bait stations (Form GCE-PC-01); bi-monthly pitfall trap check; grain temperature probe weekly.
• Licensed pest contractor: Annual review by APC-licensed contractor; report filed.

8. Traceability and Integrity (Checking Point 4.1)

Traceability System (GCE-TRACE-001):
• One step back: each silo assigned a unique ID containing grower name, paddock ID, variety, harvest year (e.g., GCE-SILO3-SMITH-PADD2-APW1-2025).
• One step forward: each dispatch docket links silo ID → transport unit rego → receiving establishment.
• Batch records: receival docket → lab test result → silo allocation → fumigation certificate → dispatch docket, all linked by lot reference.
• Records retained 2 years minimum. Digital copies in FarmWizard grain management system.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

Product flow chart: see Section 3 Process Flow Diagram.

[DEFICIENCY — NOT DOCUMENTED]
No formal failed goods procedure for Noosa Ginger & Spice Co Pty Ltd. No documented process for segregation, re-testing, downgrade, destruction or DAFF notification.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (GCE-PSP-001):
• After fumigation, grain remains in sealed silo until DAFF sampling. No grain movement between sampling and loading.
• Transport unit inspected for cleanliness and freedom from infestation before loading (Form GCE-TU-01).
• Sealed containers: seal number recorded on dispatch docket; communicated to receiving port.
• Co-mingling of export-cleared grain with non-cleared grain is prohibited at all times.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.8 | February 2028 | Manager 89 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 89 (Quality Lead)  |  Title: HACCP Lead  |  Date: February 2028

Signature:  ________________________________