| HACCP PLAN Hazard Analysis and Critical Control Point Plan Southridge Crops Pty Ltd | RE Number: RE-WA-3007 Doc Ref: SOU-HACCP-008 Version: 4.7 Date: April 2024 State: New South Wales |
| --- | --- |

1. Introduction, Scope and Product Description

Establishment: Southridge Crops Pty Ltd

Address: 8 Station Road, New South Wales

Commodity: Chickpeas (Desi, Kabuli), Red Lentils

Scope: Receival, drying, cleaning, sizing, colour sorting and packing of chickpeas and lentils for export. Registered area RE-QLD-2891, Millmerran facility.

Intended use: Chickpeas for retail and food service. Red lentils for retail soup/curry packs. Destinations: India, Pakistan, Bangladesh, Middle East.

2. HACCP Team

Manager 8 (Quality Lead)

Operator 8A

Operator 8B

3. Process Flow Diagram

| Step | Process | Input | Output |
| --- | --- | --- | --- |
| 1 | Receival / intake | Farm truck deliveries | Grain tipped into receival pit; probe sample for moisture, screenings |
| 2 | Drying (chickpeas if >13% moisture) | Wet chickpeas | Dried to ≤12.5% moisture |
| 3 | Bulk storage | Dry grain | Allocated to SILO-1 (chickpeas) or SILO-2 (lentils) |
| 4 | Pre-cleaning | Stored grain | Pre-cleaner removes coarse trash and weed seeds |
| 5 | Gravity separation | Pre-cleaned grain | Density separation removes immature seeds |
| 6 | Sizing | Gravity-separated grain | Screened to export grade size |
| 7 | Colour sorting | Sized grain | Optical colour sorter removes discoloured, mouldy seeds |
| 8 | Packing | Sorted grain | Packed in 25 kg woven bags or 500 kg bulk bags |
| 9 | Fumigation (if required) | Packed bags in sealed shed | Phosphine fumigation of export packs |
| 10 | Dispatch | Fumigated packed goods | Loaded to container; dispatch docket signed |

4. Hazard Analysis

| Step(s) | Hazard Type | Hazard Description | Prob | Sev | Risk | CCP? | Control Summary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1–3 | Chem | Mycotoxin (Aflatoxin, Ochratoxin A) from mould | 2 | 3 | 6 | CCP-1 | See HACCP Control Chart |
| 1 | Chem | Pesticide MRL exceedance | 2 | 3 | 6 | CCP-2 | See HACCP Control Chart |
| 4–7 | Phys | Foreign material — stones, metal, glass | 2 | 3 | 6 | CCP-3 | See HACCP Control Chart |
| 5–6 | Bio | Insect infestation (bruchids, weevils) in storage | 2 | 2 | 4 | — | GMP / PRP control |
| 7 | Chem | Chemical cross-contact from colour sorter lubricants | 1 | 2 | 2 | — | GMP / PRP control |
| 8–9 | Phys | Bag / bulk bag contamination (previous cargo) | 2 | 2 | 4 | — | GMP / PRP control |
| 9 | Chem | Phosphine fumigant residue | 2 | 3 | 6 | CCP-4 | See HACCP Control Chart |

5. HACCP Control Chart — Monitoring and Corrective Action

| CCP # | Process Step | Significant Hazard | Control Measure | Critical Limit | Monitoring Procedure | Frequency | Corrective Action | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-1 | Receival/Storage | Mycotoxin | Intake moisture test; aeration; temperature probe; lab test on export lots | Aflatoxin ≤4 µg/kg; OA ≤3 µg/kg | Moisture probe each lot; monthly temp probe; NATA test on flag lots | Monthly; per export consignment | Segregate; re-test; downgrade to feed if confirmed | Operations Manager |
| CCP-2 | Receival | Pesticide MRL | Spray diary review per grower; residue test on 5% of lots | MRL per India, Pakistan, Bangladesh (CODEX/strictest) | Spray records each grower; residue test flagged lots | Per grower/season | Quarantine; reject; notify grower; DAFF notification | Operations Manager |
| CCP-3 | Pre-cleaning/Sizing | Foreign material | Pre-cleaner + gravity table + optical sorter + metal detector | No metal >2 mm; no stones; no glass | Metal detector calibrated each shift; visual inspection each lot | Each shift | Stop line; remove contaminant; inspect; re-run | Operations Manager |
| CCP-4 | Fumigation | Phosphine residue | Licensed fumigator; ventilation protocol; residue test | ≤0.01 mg/kg at load-out | Licensed fumigator certificate; vent period checklist | Each fumigation event | Extend vent; re-test; do not load until cleared | Operations Manager |

6. Hygiene and Waste Controls (Checking Point 3.1)

Hygiene and Sanitation Program (DDPE-HSP-001):
• Pre-season: full clean-down of receival pit, conveyors, pre-cleaner, gravity table, colour sorter and packing lines.
• During season: colour sorter air nozzles cleaned daily; packing tables sanitised between shifts.
• Personnel: handwash on entry; no food in processing area; dust masks and gloves provided.
• Records: Cleaning Register (DDPE-CLN-01) signed after each event.

Waste Management Procedure (DDPE-WMP-001):
• Pre-cleaner trash / weed seeds: collected in sealed bins; removed to composting weekly.
• Dust: cyclone collector emptied weekly; disposed to composting.
• Reject bags: damaged or misprinted bags recycled through approved contractor.
• Chemical waste: fumigant containers disposed per APVMA label; manifest maintained.

7. Pest Control (Checking Point 3.2)

[DEFICIENCY — NOT DOCUMENTED]
No formal pest control program is documented for Southridge Crops Pty Ltd. No bait station map, monitoring schedule, contractor engagement or corrective action procedures are included.

8. Traceability and Integrity (Checking Point 4.1)

[DEFICIENCY — NOT DOCUMENTED]
This HACCP plan does not include a documented traceability system linking receival, processing, packing and dispatch.

Specifically absent:
• No lot coding system for receival batches.
• No documented one-step-back / one-step-forward traceability procedure.
• No dispatch register linking packed bags to container and destination.
• No recall procedure or trace exercise schedule.

ACTION REQUIRED: Implement a Traceability System covering:
(a) Unique lot codes at receival; (b) Processing batch records; (c) Packing lot codes on bags;
(d) Dispatch register; (e) Annual trace exercise; (f) Recall activation protocol; (g) Record retention minimum 2 years.

9. Product Flow Chart and Failed Goods (Checking Point 4.1)

Product flow chart: see Section 3 Process Flow Diagram.

Failed Goods Procedure (DDPE-FGP-001):
• Mycotoxin exceedance: lot placed on HOLD; re-test; if confirmed — downgrade to feed or destroy.
• Pesticide MRL failure: quarantine; reject export; notify grower; DAFF notified.
• Foreign material detection: stop line; remove contaminant; inspect batch; re-run.
• Non-conformance records maintained in DDPE-NCR register.

10. Phytosanitary Security / Contamination Risk (Checking Point 4.2)

Phytosanitary Security (DDPE-PSP-001):
• Export-cleared bags stored in sealed shed; access restricted.
• Fumigation certificate linked to dispatch docket.
• Container inspection before loading; seal number recorded.
• DAFF phytosanitary certificate issued per consignment.

11. Verification and Record Keeping

Internal audit of HACCP plan: annual, by HACCP team lead.

External audit: DAFF registered establishment audit (periodic).

CCP monitoring records retained minimum 2 years.

HACCP plan reviewed following: product change, process change, new hazard identified, customer complaint, DAFF non-conformance.

12. Approval and Review

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 4.7 | April 2024 | Manager 8 (Quality Lead) | Current approved version — see document history |

Approved by:  Manager 8 (Quality Lead)  |  Title: HACCP Lead  |  Date: April 2024

Signature:  ________________________________